# Survey of Compute Accelerator Hardware: Architectures, Historical Performance Models, and the AI Landscape (V5 - Final Outline)

Master Structural Outline and Complete Conceptual Taxonomy

This document presents the definitive master structural outline for our academic survey paper on compute accelerator hardware. This framework enforces strict logical progression (Workloads First) and integrates comprehensive client-side, mobile, edge, and sovereign architectural considerations alongside advanced mathematical performance modeling.

## Chapter 1: Historical Roots of Compute Acceleration

- The Von Neumann Bottleneck: CPU instruction-fetch/decode bottlenecks, program counter constraints, register file pressure, and the classic "memory wall."

- Instruction-Level Parallelism (ILP) Limits: The breakdown of Dennard scaling (leakage current, power wall), dark silicon phenomena, and Amdahl's Law governing serial execution bottlenecks.

- Vector Extensions: The architectural transition from pure scalar execution to SIMD (Single Instruction, Multiple Data) execution units, detailing instruction set extensions (MMX, SSE, AVX-512, ARM Neon).

- The GPGPU Revolution: Evolution of fixed-function graphics hardware pipelines into fully programmable unified shader processors. The creation of NVIDIA’s G80/Tesla architecture and the introduction of the CUDA computing platform.

## Chapter 2: Hardware Architectures and Execution Models (With Client-Side Extensions)

### 2.1 Control-Flow vs. Dataflow Paradigms

- Temporal (Von Neumann) Control-Flow: Instruction-driven execution where operations are fetched, decoded, and dispatched sequentially to physical ALU resources.

- Spatial Dataflow: Data-driven execution where instructions are mapped physically onto a network of computing elements, and data streams directly between elements.

### 2.2 Taxonomy of Parallelism & Hardware Paradigms

| Hardware Paradigm | Core Description | Primary Strengths | Primary Weaknesses | Most Suitable Workloads |
| --- | --- | --- | --- | --- |
| MIMD (Multicore CPUs) | Multiple instruction streams executing independently on multiple data streams. | Highly flexible; exceptional single-thread performance; advanced branch prediction. | Severe instruction-decoding overhead; low compute density per unit of silicon area. | Sequential processing, database querying, operating system control loops. |
| SIMD (Vector/GPU) | A single instruction stream broadcast across multiple data processing units. | High arithmetic throughput; dense ALU arrays; amortized instruction-fetch overhead. | Highly vulnerable to thread/branch divergence; memory coalescing overhead. | Matrix algebra, image processing, contiguous 2D grid operations, physical simulations. |
| Systolic Arrays | Data flows through a multi-dimensional grid of Processing Elements (PEs) using direct clock-step propagation. | Absurdly low register file pressure; minimal SRAM access energy; near-ideal compute density. | Extremely rigid layout; zero flexibility for irregular matrix sparsity or dynamic routing. | Dense GEMM (General Matrix Multiply) operations, convolutional neural layers. |
| FPGAs & Reconfigurable Logic | Hardware logic gates, routing pathways, and interconnects are dynamically programmed after manufacturing. | Deterministic execution latency; custom pipelines tailored to specific layouts; high power efficiency. | Low clock speeds relative to ASICs; poor floating-point performance density; complex compiler pipelines. | Real-time edge signal processing, software-defined networking, batch-size-1 streaming inference. |

### 2.3 Memory Subsystem Taxonomy & Host Orchestration

- Unified Memory Architectures (UMA) vs. Discrete VRAM:

  - Enterprise/Discrete Model: Dedicated, high-bandwidth graphics memory (HBM3e/HBM4 or GDDR6) on package. Requires high-latency host transfers over PCIe buses, incurring severe serialization copies.

  - Integrated Client SoC Model: Edge NPUs, GPUs, and CPUs share a single pool of physical system memory (e.g., LPDDR5X) on the same System-on-Chip (SoC). Bypasses redundant memory copying, but memory bandwidth is capped by the narrower shared system bus.

- Cache Management Taxonomies: Hardware-managed hierarchical cache architectures (coherent L1/L2/L3) vs. Direct-Managed Scratchpad Memories (software-controlled static RAM requiring developer/compiler DMA orchestration).

- Host-to-Accelerator Polling and Synchronization Overhead: Dissecting the energy costs of task orchestration at the edge. How CPU remote procedure call (RPC) polling, driver-level synchronization, and sleep-latency timing mismatches cause the host CPU to consume up to 30% of total system energy during active NPU runs.

### 2.4 Client-Side Execution Taxonomies

- Precision-Flexibility Trade-off: Contrast of enterprise datatype diversity (MX formats, dynamic FP8/FP6/FP4) with spatial edge layouts strictly optimized for static low-power integer arithmetic (INT8/INT4). Issues of GPU/CPU runtime fallbacks when running non-standard activation shapes.

- Heterogeneous Task Partitioning: Execution engine division of labor on modern SoCs: CPU optimization for local non-vector branch math, GPU scaling for large-batch operations, and edge NPU scaling for memory-bound, low-batch matrix-vector operations.

## Chapter 3: Analytical Performance Modeling

- The Roofline Model: Visualizing compute-bound versus memory-bandwidth-bound execution phases.

- Mathematical Formulations:

  - Operational (Arithmetic) Intensity:
I = FLOPs / Bytes transferred

  - Attainable Performance:
P = min(P_peak, I * b)where b represents off-chip memory bandwidth.

- Quantization and Data-Type Scaling (Edge Impact): Mathematical modeling of how moving from high-precision formats to FP16, FP8, INT4, INT3, or activation-only quantization shifts a workload's arithmetic intensity to the left on the Roofline curve, lowering bandwidth demand by 4x and shifting bottlenecks from memory-bound to compute-bound to fit tight edge RAM constraints.

- Workload Execution Phase Divergence: Modeling prefill/prompt phase (compute-bound, mapping to matrix engines) vs. autoregressive decode phase (extremely memory-bandwidth-bound, dependent on rapid KV-cache DRAM-to-SRAM retrieval).

- The "Exploitable Configuration Slack" Phenomenon: Theoretical modeling of edge quality of experience (QoE) limits. Once token generation exceeds human reading speed (10–15 tokens/sec), throughput gains offer diminishing returns. Under this limit, Dynamic Voltage and Frequency Scaling (DVFS) can be used to throttle NPU/DDR clock speeds, saving up to 65% energy on phones and 12% on laptops while preventing thermal throttling.

- NPU Compiler Bucketing and Latency Non-linearity: Mobile NPU reliance on static compilers that use pre-determined execution context "buckets" (e.g., 128, 256, 512, 1024 tokens) to handle input lengths. Analysis of step-function latency spikes when model context footprints cross static bucket thresholds at runtime.

- The Tokens-per-Joule (Tokens/J) and SWaP-C Framework:
Energy Efficiency = Total Tokens Generated / System Energy Consumed [Tokens/Joules]Establishing SWaP-C (Size, Weight, Power, and Cost) as the ultimate evaluation matrix for edge platforms.

## Chapter 4: Mapping AI Workloads to Silicon (Workloads First)

### 4.1 Modern Deep Learning Bottlenecks (Enterprise)

- Prefill vs. Decode Divergence: Mapping highly compute-bound prompt evaluation versus highly memory-bandwidth-bound autoregressive decoding to hardware.

- Sparsity & MoE Failure Modes on GPUs: How dynamic, unstructured sparsity (e.g., Mixture of Experts) causes severe thread divergence, uncoalesced memory reads, and inter-chip "all-to-all" network congestion on traditional dense SIMD GPUs.

- The KV-Cache Footprint: Quadratic capacity growth of activation memories in long-context model runs, starving processing units.

- Transformer Execution Optimization: Multi-Head Attention (MHA) vs. Multi-Query Attention (MQA) vs. FlashAttention spatial optimization techniques.

- Parallelism Topologies: Scaling models across multiple chips using Data Parallelism (DP), Tensor Parallelism (TP), Pipeline Parallelism (PP), and Fully Sharded Data Parallel (FSDP).

- Sparsity Exploitation: Dissecting structured 2:4 sparsity in modern Tensor Cores and dynamic execution gating mechanisms.

### 4.2 Client-Side Workloads & Low-Power Constraints

- The Battery & Thermal Envelope: Running inference under tight power limits (e.g., 35W laptop vs. 5W smartphone) compared to a data center node.

- Small Language Models (SLMs) on the Edge: Extreme quantization (INT4/INT3/activation-only quantization) to fit models (Phi-4, Llama-3-8B, Gemma-2) into limited edge RAM.

- Ambient Background Inference: Low-power partitioning—routing background wake-word detection, eye-tracking, or real-time transcription to dedicated, single-digit milliwatt NPUs while keeping the GPU/CPU completely asleep.

### 4.3 Physical World AI (Embodied Intelligence & Robotics)

- Real-time State-Space Modeling (SSMs): Execution profiles of low-latency, batch-size-1 state estimation and tracking models (e.g., Mamba configurations).

- Heterogeneous Multimodal Sensor Fusion: Managing synchronous, high-velocity incoming streams (LiDAR, IMU, stereo cameras) on shared, non-copying memory planes.

- Non-linear Trajectory & Physics Optimization: The strict computational need for double-precision math (FP64) and graph-based planning, which are highly inefficient on low-precision AI matrix hardware.

## Chapter 5: Enterprise Hardware and the Startup Landscape

### 5.1 The Hyperscale Powerhouses

- NVIDIA: Blackwell (B200, GB200 NVL72) and the upcoming Rubin architecture. Microarchitectural analysis of second-generation Transformer Engines, decompression engines, and NVLink 5 scale-up topologies.

- AMD: CDNA3/4 Instinct lines (MI300X, MI325X, MI350). Microarchitectural focus on giant consolidated memory pools (HBM3e/HBM4) and chiplet-based physical layouts.

- Google: TPU v5p and TPU v6 (Trillium). Dissecting custom systolic array structures and scale-out architecture using optical circuit switching (OCS).

- Amazon (AWS): Trainium 2/3 and Inferentia 2 architectures. Custom silicon built specifically for high-efficiency, scale-out training and inference in EC2 clusters.

- Meta: MTIA (Meta Training and Inference Accelerator). Low-power custom ASICs built specifically for recommendation model inference and ranking pipelines.

- Microsoft Custom Silicon: Maia 100 & Maia 200 (TSMC 3nm). Tile-based designs, Tile Tensor Units (TTU) for low-precision math, Tile Vector Processors (TVP), and software-managed hierarchical SRAM structures (CSRAM/TSRAM) with custom ATL interconnects.

- OpenAI Custom Silicon: "Jalapeño" ASIC (Co-Developed with Broadcom). High-throughput token decoding, tailored memory pathways, and stripped-out training scheduling blocks.

### 5.2 The Startup Challengers

- Cerebras: Wafer-Scale Engine 3 (WSE-3). Analysis of a single monolithic 46,225 mm² wafer, featuring 4 trillion transistors, 900,000 AI-optimized cores, and 44 GB of physical on-chip SRAM.

- Groq: Language Processing Unit (LPU). An entirely static, compiler-scheduled, SRAM-only design (500 MB per chip) designed to provide deterministic, ultra-low-latency real-time inference.

- SambaNova: Reconfigurable Dataflow Unit (RDU). Analyzing spatial architectures, dynamic routing, and multi-tile layouts optimized for massive-context execution.

- Tenstorrent: Wormhole and Galaxy architectures. Open-source RISC-V compute cores paired with a packet-switched Network-on-Chip (NoC) to democratize custom AI execution fabrics.

## Chapter 6: Client-Side AI Accelerators (Edge, PC, and Mobile)

### 6.1 The AI PC Platform

- Microsoft's Copilot+ specification requiring a minimum of 40 NPU TOPS.

### 6.2 Silicon Architectures at the Client Edge

- Qualcomm: Snapdragon X Elite and X2 Elite (Hexagon NPU delivering up to 80-85 NPU TOPS, specialized in low-power integer arithmetic).

- Intel: Lunar Lake (NPU4) and Panther Lake (18A process node, NPU5 hitting up to 50 TOPS).

- AMD Client: Ryzen AI 300/400 (XDNA 2 architecture, delivering up to 60 TOPS using spatial dataflow tiles).

- Apple: Apple Silicon (A18/A19/M4/M5). Neural Engine evolution vs. the unified memory architecture (UMA) enabling desktop-class LLMs to fit within integrated memory pools.

- Discrete Client Hardware: The critical role of local NVIDIA RTX Tensor Cores (Ada Lovelace, Blackwell desktop variants) for local model training, high-precision scientific computing, and professional creative workflows.

### 6.3 Worldwide AI Hardware Startup Map (2026 Landscape)

A comprehensive landscape of startups competing in the compute accelerator space outside of traditional US giants:

- Axelera AI (Netherlands): Focus on high-performance Digital In-Memory Computing (D-IMC) using RISC-V edge execution.

- Semron (Germany): Pioneering "CapRAM" technology, utilizing 3D-stacked memcapacitors to store weights via electric fields.

- SpiNNcloud / SpiNNaker2 (Germany/UK): Asynchronous, event-driven neuromorphic cloud infrastructure using packet-switching.

- Cortical.io (Austria): "Semantic folding" technology utilizing FPGAs to accelerate NLP workflows.

- Cambricon Technologies (China): High-end ASICs highly integrated with domestic generative architectures.

- Moore Threads (China): Domestic general-purpose GPUs utilizing "Huagang" architecture for massive GPGPU training clusters.

- Biren Technology (China): Developers of the BR series high-end general-purpose GPGPUs.

- Muxi Technology (China): Training cluster configurations focused on chaining tens of thousands of local GPUs.

- Tsingmicro (China): Reconfigurable data-flow computing architectures.

- Preferred Networks / PFN (Japan): Custom MN-Core series accelerators optimized for ultra-high-density matrix math.

- Sakana AI (Japan): Specialized model synthesis engines driving demand for alternative domestic hardware.

- Rebellions (South Korea): ASICs optimized for real-time financial trading, computer vision, and low-power LLM inference.

- FuriosaAI (South Korea): Data-center-scale inference architectures optimized for Vision Transformers and LLMs.

### 6.4 The Top 5 Global Contenders

- Preferred Networks (Japan): Scaled custom MN-Core silicon designed specifically to bypass GPU bottlenecks in complex physics and modeling workloads.

- Rebellions (South Korea): Production-ready, ultra-low-latency enterprise and edge-LLM inference solutions.

- Cambricon Technologies (China): Deep domestic vertical integration and massive local software/hardware ecosystem footprint.

- Axelera AI (Netherlands): High-density Digital In-Memory Computing delivering spatial edge acceleration for European systems.

- Semron (Germany): Disruptive "CapRAM" physical device design bypasses transistors to deliver 20x energy step-changes for client form factors.

## Chapter 7: Software Stacks, Compilers, and Ecosystems

- The Software Moat: NVIDIA CUDA, TensorRT, Triton, and native deep integration with PyTorch.

- Hardware Abstraction Layers: AMD's ROCm (HIP, MIGraphX) and Intel's oneAPI (OpenVINO).

- Intermediate Representations (IR) and JIT Compilers: The critical role of MLIR, LLVM, and Google’s XLA compiler in translating high-level code to hardware-specific machine instructions.

- Software-Defined Hardware: How static scheduling compilers (such as Groq's compiler) replace hardware-level branch prediction and dynamic scheduling logic with software compile-time deterministic routing.

## Chapter 8: Hardware Co-Design & Interconnects

- Memory Wall Countermeasures: High Bandwidth Memory (HBM3e/HBM4) vs. Low-Power DDR (LPDDR5X) packaging on client-side platforms.

- High-Speed Interconnects: NVIDIA NVLink (5th Gen, 1.8 TB/s bidirectional), AMD Infinity Fabric, and optical circuit switching (OCS).

- Scale-Out Networking: Ultra-Ethernet Consortium (UEC) and InfiniBand fabrics.

## Chapter 9: Physical, Manufacturing, and Economic Limits

- Reticle Limits and Chiplets: Monolithic die size limits (858 mm²) and the physical necessity of Multi-Chip Modules (MCM).

- Advanced Packaging Bottlenecks: CoWoS (Chip-on-Wafer-on-Substrate) packaging and wafer yield dynamics.

- Thermal Design Power (TDP): Transitioning from 400W air-cooled accelerators to 1200W+ liquid-cooled supercomputing configurations.

- Economic Metrics: Total Cost of Ownership (TCO), performance-per-watt-per-dollar, and yield dynamics.

## Chapter 10: Future Horizons

- Silicon Photonics: Utilizing light-based data transmission to bypass standard copper RC delay and thermal limits.

- Processing-in-Memory (PIM): Performing fundamental arithmetic operations directly inside DRAM or SRAM structures to eliminate physical bus movement entirely.

- Alternative Architectures: Neuromorphic computing, analog AI processors, and superconducting electronics.

- Moonshot Integrations: IBM’s Co-Packaged Optics (CPO) milestone utilizing high-density mass-producible Polymer Optical Waveguides (PWGs) to scale chip interconnect bandwidth by up to 80x while reducing power budgets by 80%.

## Addendum: Technical Deep Dives (Optional Reading)

### Addendum A: GPGPU Execution Pipeline Mechanics

- Detailed physical breakdown of Unified Shader pipelines (Streaming Multiprocessors, Warps, Coalesced memory-access hardware, Register files).

- Microarchitectural execution of hardware scheduling blocks and branch-handling penalty mitigation.

### Addendum B: Systolic Array Physical Dataflow Dynamics

- Detailed spatial physical diagrams and gate-level flows: Weight-stationary vs. Activation-stationary vs. Output-stationary data movements.

- Physical register-shunting pathways, accumulation loops, and hardware implementations of Processing Elements.

### Addendum C: Microsoft FPGA Cloud Architectures (Project Catapult & Brainwave)

- Project Catapult: Detailed physical schematic of FPGA placement inline with PCIe/Ethernet interfaces on server boards, and low-latency hardware-level packet injection.

- Project Brainwave: Hard macro logic mappings of deep GRUs and spatial convolutional structures directly onto FPGA physical routing resources.

### Addendum D: Microarchitectural Dissections of Modern Custom Platforms

- Deep physical analyses of Microsoft Maia 100/200 Tile microarchitecture, Google TPU Matrix Multiply Units (MXUs), and OpenAI "Jalapeño" tailored memory controller designs.

- Hardware execution pathways of startup silicon: Cerebras WSE-3 on-wafer power delivery, Groq LPU's static compiler-controlled register buffers, and SambaNova's reconfigurable dynamic routing tiles.